# Old biomass resource categories — Finland 2035

Reference configuration BEFORE the WOOD supply-curve split (single WOOD resource). Use this as the 'before' state to explain the change to the stepwise WOOD_FI1..WOOD_FI5 ladder.

Parameter provenance (three input files):

avail_local, c_op_local  ->  Data/2035/FI/Resources.csv  (region-specific override)

avail_exterior, gwp_op_local (direct+indirect, local)  ->  Data/2035/02_REF_REGION/Resources.csv

c_op_exterior (price of exterior), gwp_op_exterior (direct+indirect, exterior), co2_net (direct emissions)  ->  Data/2035/00_INDEP/Resources_indep.csv

## Parameters and units

| Resource | avail_local [GWh/y] | c_op_local [Meuro/GWh] (= EUR/MWh) | gwp_op_local direct+indirect [ktCO2-eq/GWh] | co2_net direct [ktCO2-eq/GWh] | avail_exterior [GWh/y] | c_op_exterior price exterior [Meuro/GWh] | gwp_op_exterior direct+indirect [ktCO2-eq/GWh] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| WOOD | 110,805.7 | 0.022084 (22.08) | 0.02456 | 0.000 | 0.0 | 0.000000 | 0.02456 |
| WET_BIOMASS | 1,450.6 | 0.033104 (33.10) | 0.01056 | 0.000 | 0.0 | 0.000000 | 0.01056 |
| ENERGY_CROPS_2 | 7,754.1 | 0.023524 (23.52) | 0.00756 | 0.000 | 0.0 | 0.000000 | 0.00756 |
| BIOMASS_RESIDUES | 4,985.2 | 0.013135 (13.14) | 0.00324 | 0.000 | 0.0 | 0.000000 | 0.00324 |
| BIOWASTE | 4,720.9 | 0.000112 (0.11) | 0.01488 | 0.000 | 0.0 | 0.000000 | 0.01488 |
| WASTE | 11,095.0 | 0.006079 (6.08) | 0.15010 | 0.260 | 0.0 | 0.023081 | 0.15010 |

Notes: (1) All domestic biomass has avail_exterior = 0, so imports are disabled and the exterior price/emission columns are not actually used by the optimiser (shown for completeness). (2) c_op_local is the marginal supply cost; 1 Meuro/GWh = 1000 EUR/MWh. (3) Biomass direct combustion CO2 (co2_net) is 0 because biogenic carbon is treated as neutral; only WASTE carries a non-zero direct factor. (4) gwp_op_local captures the supply-chain (harvesting, processing, transport) life-cycle emissions.

## Definition of each category

| Resource | Physical definition |
| --- | --- |
| WOOD | Lignocellulosic woody biomass. Forest-origin products and residues: pulp/sawmill by-products (black liquor, bark, sawdust), logging residues (branches, tops, stumps), secondary woodchips, direct fuelwood and landscape-care wood. Feeds the WOOD layer (boilers, CHP, gasification, wood-to-fuels). ENSPRESO codes MINBIOWOOa + MINBIOFRSR1 + MINBIOWOOW1(+a) + MINBIOWOO + MINBIOFRSR1a. *** This is the single resource later split into WOOD_FI1..WOOD_FI5. *** |
| WET_BIOMASS | Wet organic biomass (sewage sludge, manure, wet organic waste) used by anaerobic digestion / biomethanation to produce GAS. ENSPRESO code MINBIOSLU1 (sludge). Feeds the WET_BIOMASS layer. |
| ENERGY_CROPS_2 | Lignocellulosic energy crops grown on dedicated land: miscanthus, switchgrass and short-rotation coppice (willow). ENSPRESO codes MINBIOCRP31 + MINBIOCRP41. Not affected by forest-management policy. |
| BIOMASS_RESIDUES | Agricultural residues (cereal straw, stover) - NOT forest residues. ENSPRESO code MINBIOAGRW1. Feeds the BIOWASTE layer. Independent of forest management; this is the category that is sometimes confused with logging residues (which are inside WOOD). |
| BIOWASTE | Biodegradable municipal and organic waste directed to energy recovery (anaerobic digestion / biowaste streams). ENSPRESO code MINBIOMUN1. Feeds the BIOWASTE layer. |
| WASTE | Municipal solid waste for incineration (non-biogenic fraction). Classified as 'Other non-renewable' rather than biomass: it carries non-zero DIRECT combustion CO2 (0.26 ktCO2-eq/GWh). Included here for completeness. Feeds the WASTE layer. |