Supplementary Material

# Supplementary Data

Table A.1. List of lignocellulosic biomass energy conversion technologies integrated in the model with related data

| Technologies | Investment cost (€/kW of final output) | Maintenance cost (€/kW of final output/year) | Life-time (year) | Yearly capacity factor (%) | Fuel efficiency (%) | Electric efficiency (%) | Heat efficiency (%) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Pyrolysis for SLF | 1,366 | 27 | 25 | 93% | 58% | - | - |
| Pyrolysis with external H2 input for SLF (a) | 1,366 | 27 | 25 | 93% | 64% | - | - |
| Hydrolysis and fermentation for SLF | 2,405 | 57 | 25 | 96% | 38% | 5% | - |
| Gasification for SLF | 3,760 | 103 | 25 | 96% | 45% | 6% | - |
| Gasification with external H2 input for SLF (a) | 3,760 | 103 | 25 | 96% | 59% | - | 16% |
| Gasification for SNG | 2,432 | 40 | 20 | 94% | 65% | - | 22% |
| Gasification with external hydrogen input for SNG (a) | 2,432 | 40 | 20 | 94% | 79% | - | - |
| Gasification for H2 | 2,525 | 178 | 25 | 86% | 43% | - | - |
| Power-only | 1,822 | 109 | 30 | 86% | - | 32% | - |
| CHP (industrial for HT heat) | 1,081 | 40 | 25 | 85% | - | 18% | 53% |
| CHP (for DHN) | 1,081 | 40 | 25 | 85% | - | 18% | 53% |
| Technologies | Investment cost (€/kW of final output) | Maintenance cost (€/kW of final output/year) | Life-time (year) | Yearly capacity factor (%) | Fuel efficiency (%) | Electric efficiency (%) | Heat efficiency (%) |
| Boilers (industrial for HT heat) | 115 | 2 | 17 | 90% | - | - | 86% |
| Boilers (dec. for low temperature heat) | 462 | 16 | 17 | 100% | - | - | 85% |
| Boilers (DHN) | 115 | 2 | 17 | 90% | - | - | 86% |

- For the combination of biomass and hydrogen the costs of investment and maintenance were assumed similar to the technology with no hydrogen combination (i.e. biomass only).

All the prices are in 2015€.

SLF: synthetic liquid fuel; SNG: synthetic natural gas; HT: heat temperature; DHN: district heating network; dec.: decentralised.

# References

Albrecht, F. G., König, D. H., Baucks, N., and Dietrich, R. (2017). A standardized methodology for the techno-economic evaluation of alternative fuels – A case study. Fuel 194, 511–526. doi:10.1016/j.fuel.2016.12.003.

Danish Energy Agency, and Energinet (2017). Technology Data for Renewable Fuels. 1–259.

Gironès, V. C., Moret, S., Peduzzi, E., Nasato, M., and Maréchal, F. (2016). Optimal use of biomass in large-scale energy systems: Insights for energy policy. ECOS 2016 - Proc. 29th Int. Conf. Effic. Cost, Optimisation, Simul. Environ. Impact Energy Syst.

IRENA (2020). Renewable Power Generation Costs In 2019. Abu Dhabi.

Limpens, G., Jeanmart, H., and Maréchal, F. (2020). Application of EnergyScope TD to the Belgium case Supplementary Material. 1–49.

Prussi, M., Yugo, M., De Prada, L., Padella, M., Edwards, M., and Lonza, L. (2020). JEC Well-to-Tank report v5. doi:10.2760/959137.